# Kyle Keislar

# ECE524/L FPGA/ASIC Design and Optimization Using VHDL Lab

# Lab 3

# FPAG Features Clock Management, DSP Blocks, DDR, and SRL

# 10/09/2020

## Introduction & Problem Statements

In this experiment a basic interleaver was designed. The interleaver combines data from two sources (P0, P1) by measure of a blending factor( alpha and its compliment). The output of the interleaver is then the sum of the products of its two sources and blending factor. The behavior of the interleaver is described in equaition 3.1 below

Eq 3.1: Interleaver Behavior

To explore FPGA utilizations and resource mapping, multiple implementations of the fundamental interleaver can be derived. In figures 3.0A-C, increasing complex iterations of the design are realized at the high level.

Fig 3.0.A: Signal Stream Sender High-Level Block Diagram

Fig 3.0.B: Signal Stream Interleaver High-Level Block Diagram

Fig 3.0.B: Signal Stream Interleaver High-Level Block Diagram

The code for the synthesized design of each of these files can be found in the Appendix below.

An implemented design of this filter was not realized using the Zedboard for this experiment.

## Procedure:

Part I (Direct Implementation)

Task 1: Write VHDL code to implement the circuit shown in Figure 3.1. In order to verify the design, you can apply two sine waves on P0 and P1 with different amplitude. Change alpha value from 0 to 1. The blend output should make transition from P0 to P1 waveform. You should test your design for several cases in order to prove the correct functionality. For instance, case of alpha = 1 or alpha = 0 to show the output shows either one of the waveforms. A more interesting simulation case should be included to prove the functionality for a linear transition of alpha from 0 to 1. Assume P 0 and P1 are 8 bit values and 0≼ alpha ≼1. Prove that your design is working by providing the simulation.

The VHDL test benches and design for this task can be seen below in the Appendix as items A.1 and A.2.

Part II (Optimized Implementation)

Task 2: For the optimized implementation, there are two clock domains in this design. Instantiate the clock manager module with 50 MHz clock input and deskewed output and use the clk2x output of the clock manager to generate 100 MHz clock for your DSP block. Write complete VHDL code to infer all blocks (other than DCM). Prove that your design is working by providing the simulation. You can use the testbench you developed in the first part of the lab.

The VHDL test benches and design for this task can be seen below in the Appendix as items A.1 and A.3.

Part III (Equalizing delays)

Task 3: : In the third part of the lab, apply 64 clock cycle delay to P 0 stream using SRL components according to Figure 3.3. You can use IP Integrator to generate the SRL components. The testbench developed in the first part of the lab should be slightly modified so you can apply it to the third part of the lab. Provide the simulation waveform to prove your design is functioning.

The VHDL test benches and design for this task can be seen below in the Appendix as items A.4 and A.5. The synthesizable shift register used in this portion of the experiment was utilized from the Xilnix Synthesis Guide and can be seen in the Appendix as item A.6.

Task 4: : Replace the SRL components with Block RAMs (BRAMs) that create the same amount of delay and prove your design works by providing simulation.

## Results (Data):

## Waveforms graphed from generated & simulated data:

Fig 3.1: Direct Implementation: Alpha= 1

Fig 3.2: Direct Implementation: Alpha changing from 1 to 0

Fig 3.3: Direct Implementation: Alpha= 0

Fig 3.4: Optimized Implementation: Alpha= 1; Clock Sync Delay

Fig 3.5: Optimized Implementation: Alpha= 1; Clocks Synced

Fig 3.6: Optimized Implementation: Alpha changing from 1 to 0

Fig 3.7: Optimized Implementation: Alpha= 0

Fig 3.8: Equalized Interleaver Delay: P_0 input read

Fig 3.8: Equalized Interleaver Delay: delayed P_f output calculated from P_0

## Analysis:

The simulated design for this experiment successfully demonstrates the functionality of the interleaver in experimental tasks 1 and 2. Task 3 was only partially functional, but it can be concluded that the functionally of the Interleaver was preserved in this implementation.

Figure 3.1 shows the initial output of the direct implementation of the interleaver. This output represents the sum product of P_0, P_1, alpha and 1-alpha when alpha is 1. The output waveform, P_f, is comprised entirely of data from P_0 and is shown to mimic this input. As the simulation progresses, Figure 3.2 shows how the output values can be altered when alpha is changed from 1 to 0 at a linear rate. After five clock pulses, it can be seen that the output has shifted from being composed of the single magnitude sinusoid input P_0 to the double input sinusoid P_1. Figure 3.3 shows the output after alpha has changed being passed values from input P_1. This outcome confirms the working condition of the direct implementation as an blending value of 0 nullifies either of the inputs while a blending value of 1 passes the inputs.

Figure 3.5, 3.6 and 3.7 show a similar conclusion in the optimized implementation of the interleaver from task 1. This implementation utilized the DSP blocks of the FPGA in its synthesized design, resulting in a more efficient use of resources. This is proved by the addition of a second clock (clk_2) which was constructed using the Vivado clocking wizard to pulse at twice the rate of the original input clock (50 MHZ). In order to synchronize these locks and remove the maximum error possible, the clocking wizard was allotted a total of 38 clock cycles to provide enough input to synchronize. This delay can be seen in Figure 3.4 above and was accounted for in the Task 2 testbench.

Finally in Figure 3.8 and 3.9 a Equalized dealy implementation of the interleaver can be shown to have partial functionality. The original input for one of the blending factors (alpha) and the input values (P_0) was delayed by 64 clock cycles to provide an instantaneous output. Figure 3.8 shows the first two read input values of P_0 after the clocks were synchronized (at clk_cnt=64). Figure 3.9 shows the delayed value of the second input of P_0 64 clock cycles later. The output values at this time (~3020 ns) equals -19.0. Given the value of alpha, by Eq 3.1 the theoretical values of P_0 and P_1 at this time can be derived. These are:

PF (t=3020)=P0*α+P1*1-α= -19.0

PF(t=3020) =0.5*P0(t=3020)+1-0.5*P1(t=3020)

Values delayed value of P_0 and current value of P_1 at 3020ns can be observed form the waveform in Figure 3.9 as signals P_0_temp and P_1 respectively. P_0 is shown to be 38, and P_1 is shown to be -76. When these numbers are plugged into the Equation above theoretical value can be calculated

PF(t=3020) =0.5*(38)+1-0.5*(-76)

PF(t=3020) =19+ -38

PF(t=3020) =-19

It should be noted that the testbench used for this task did not provide the adequate delays when reading in inputs from file. Further iterations of this equalized delay implementiaon are needed to provide a waveform more representative of the design’s functionality.

APPENDIX

Questions

- 1) (Part I: Direct Implementation)  Implement the design. What is the resource utilization of the design?

Utilization report

- 2) Have multipliers and adder been mapped to the DSP block? If not, can you force the tool to map these components to the DSP blocks? How do you prove your answers?

No the multiplexers are not mapped to any DSP blocks. This can be seen in the implemented design utilization report

In order to map the multiplier and adder to the DSP block this specific board component can be inferred by the synthesis tool using a predefined synthesis attribute use_dsp in the .vhd design file for the interleaver (lab3_partI.vhd). To satisfy the computational requirements of the interleaver, 3 DSP blocks would need be used. One to calculate P0*alpha, another to calculate P1*(1-alpha) and a third to add these two products to generate Pf.

- 3) (Part II: Optimized Implementation)  Implement the design. What is the resource utilization of the design?

Utilization report

- 4) Have multipliers and adder been mapped to the DSP block? If not, can you force the tool to map these components to the DSP blocks? How do you prove your answers?

Yes, The implemented utilization report  lists one DSP48 block being used on the ZedBoard. This confers with the arithmetic operations performed in a clock cycle (1 multiplication and 1 addition).

- 5) (Part III: Equalizing Delays; SRL)  Implement the design. What is the resource utilization of the design?

Utilization report

- 6) Can you implement the same delay using Block RAM (BRAM)? How? Explain your answer.

Yes. A BRAM implementation could be done by writing a data stream of bit values of P_0 and alpha to any memory address. A functional delay can then be realized by reading these values form a memory address 64 indicies higher than the original written address. A dual port ram could be utilized to read and write alpha, P_0 to different address ranges.

- 7) (Part III: Equalizing Delays; BRAM)  Implement the design. What is the resource utilization of the design?

Utilization report

- Tb_Lab3_PartI.vhd

----------------------------------------------------------------------------------

-- Company:

-- Engineer:

--

-- Create Date: 10/08/2020 03:14:49 PM

-- Design Name:

-- Module Name: tb_lab3_PartI - Behavioral

-- Project Name:

-- Target Devices:

-- Tool Versions:

-- Description:

--

-- Dependencies:

--

-- Revision:

-- Revision 0.01 - File Created

-- Additional Comments:

--

----------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use ieee.math_real.all;

use std.textio.all;

use ieee.std_logic_textio.all;

use ieee.std_logic_unsigned.all;

entity tb_lab3_PartI is

--  Port ( );

end tb_lab3_PartI;

architecture Behavioral of tb_lab3_PartI is

--Constant Definitions

CONSTANT CP: TIME := 125ns;--clock period

CONSTANT ALPHA_VAL: real := 0.00;--blend value

CONSTANT ONE_ALPHA_VAL: real := 1.00-ALPHA_VAL; --blend value inverse

CONSTANT A_LEN: integer := 8;       --blend value bit length

CONSTANT P_LEN: integer := 8;       --pixel input value bit length

CONSTANT SHIFT_VAL: real := 2**real(A_LEN-1)-0.01; --real value representing the # of bits to shift left (arithmetic shift with real inputs)

--not synthesizable, excludes sign bit shift

--Signal Definitions

signal p0,p1: std_logic_vector(7 downto 0) := (Others =>'0');

signal a,a_chg: std_logic_vector(7 downto 0) := (Others =>'0'); --amount ot change alpha by

signal a1,a1_chg: std_logic_vector(7 downto 0) := (Others =>'0'); --amount to change one allpha by

signal pf: std_logic_vector(15 downto 0) := (Others =>'0');

SIGNAL PULSE: TIME:=CP*0.5;--pulse width

signal m_clk: std_logic := '0'; --master clock, filter reset

signal  chg_cnt: integer := 0; --used to read in files, gradually change alpha in Part I test bench

Component lab3_partI

Port(P_0: in STD_LOGIC_VECTOR (7 downto 0); --first pixel data stream input

P_1: in STD_LOGIC_VECTOR (7 downto 0); --second pixel data stream input

alpha: in STD_LOGIC_VECTOR (7 downto 0); --blending factor

one_alpha: in STD_LOGIC_VECTOR(7 downto 0);-- inverse blending factor

P_f: out STD_LOGIC_VECTOR(15 downto 0));--Output equal to :: P_0*alpha + P_1*one_alpha

end component;

begin

uut: lab3_partI--Signal Stream Blender

Port Map( P_0 => p0,

P_1 => p1,

alpha =>a,

one_alpha=>a1,

P_f =>pf);

--Clock Processs

m_clock:process --free running clock

begin

m_clk <= '1';

wait for PULSE ;

m_clk <= '0';

wait for PULSE;

end process;

--Test Vectors

process (m_clk)

variable a_calc, a1_calc: real :=0.00; --used to calculate gradual shift of alpha values

begin

if(m_clk'event AND m_clk='1') then

chg_cnt <= chg_cnt +1;

end if;

if(chg_cnt<=12) then --after 12 clock pulses (value chosen to show previous perodicity of sin wave P0)

a_calc := 0.00;

a1_calc:=SHIFT_VAL;

a_chg<=std_logic_vector(to_unsigned(natural(a_calc),A_LEN)); --alpha is 0

a1_chg<=std_logic_vector(to_unsigned(natural(a1_calc),A_LEN)); --one_alpha is 1

elsif(a_chg <"10000000") then

a_calc := a_calc +(SHIFT_VAL/5.00);

a1_calc:=a1_calc -(SHIFT_VAL/5.00);

a_chg<=std_logic_vector(to_unsigned(natural(a_calc),A_LEN)); --alpha is changing towards 1

a1_chg<=std_logic_vector(to_unsigned(natural(a1_calc),A_LEN)); --one_alpha is changing towards 0

elsif(a_chg >="10000000") then

a_calc := SHIFT_VAL;

a1_calc:=0.00;

a_chg<=std_logic_vector(to_unsigned(natural(a_calc),A_LEN)); --alpha is 1

a1_chg<=std_logic_vector(to_unsigned(natural(a1_calc),A_LEN)); --one_alpha is 0

end if;

a<= a_chg;

a1<= a1_chg;

end process;

P0_read_in:process(m_clk)--reads in data for P0

file TVs : text open read_mode is "P0_in.txt";-- "8_mhz_dat.txt"; --16_mhz_dat.txt -- 24_mhz_dat.txt

variable row_cnt: integer := 0;--row to read from

variable read_in     : line;

variable read_from_line : integer;

variable r_temp : std_logic_vector(7 downto 0);

begin

if(m_clk'event AND m_clk='1') then

if(not endfile(TVs)) then

row_cnt := row_cnt + 1;

readline(TVs,read_in); --read line from file

end if;

read(read_in,read_from_line); --read read line data from file into bit array

read(read_in,r_temp); --read read line data from file into bit array a

P0   <= std_logic_vector(to_signed(read_from_line,P_LEN));

end if;

end process;

P1_read_in:process(m_clk)

file TVs : text open read_mode is "P1_in.txt";-- "8_mhz_dat.txt"; --16_mhz_dat.txt -- 24_mhz_dat.txt

variable row_cnt: integer := 0;--row to read from

variable read_in: line;

variable read_from_line : integer;

variable r_temp : std_logic_vector(7 downto 0);

begin

if(m_clk'event AND m_clk='1') then

if(not endfile(TVs)) then

row_cnt := row_cnt + 1;

readline(TVs,read_in); --read line from file

end if;

read(read_in,read_from_line); --read read line data from file into bit array

read(read_in,r_temp); --read read line data from file into bit array a

P1   <= std_logic_vector(to_signed(read_from_line,P_LEN));

end if;

end process;

Pf_write:process

CONSTANT COL: INTEGER :=1;

type t_integer_array is array(integer range <> )  of integer;

file Output:text open write_mode is "Pf_out.txt";

variable write_in: line; --reads in a line from the text file

variable write_line_to_file: t_integer_array(1 to COL);

begin

write_line_to_file(1):= to_integer(signed(pf));

write(write_in,write_line_to_file(1));

writeline(Output, write_in);

wait for CP;

end process;

end Behavioral;

- Lab3_PartI.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_signed.all;

entity lab3_partI is

Port(P_0: in STD_LOGIC_VECTOR (7 downto 0); --first pixel data stream input

P_1: in STD_LOGIC_VECTOR (7 downto 0); --second pixel data stream input

alpha: in STD_LOGIC_VECTOR (7 downto 0); --blending factor

one_alpha: in STD_LOGIC_VECTOR(7 downto 0);-- inverse blending factor

P_f: out STD_LOGIC_VECTOR(15 downto 0));--Output equal to :: P_0*alpha + P_1*one_alpha

end lab3_partI;

architecture Behavioral of lab3_partI is

signal temp0,temp1:std_logic_vector(15 downto 0) := (Others =>'0');

begin

temp0 <= P_0*alpha;

temp1 <= P_1*one_alpha;

P_f <= P_0*alpha + P_1*one_alpha;

end Behavioral;

- Lab3_PartII.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_signed.all;

entity DSP_inter is

Port(P_0: in STD_LOGIC_VECTOR (7 downto 0); --first pixel data stream input

P_1: in STD_LOGIC_VECTOR (7 downto 0); --second pixel data stream input

alpha: in STD_LOGIC_VECTOR (7 downto 0); --blending factor

one_alpha: in STD_LOGIC_VECTOR(7 downto 0);-- inverse blending factor

clk: in STD_LOGIC;

P_f: out STD_LOGIC_VECTOR(15 downto 0));--Output equal to :: P_0*alpha + P_1*one_alpha

end DSP_inter;

architecture Behavioral of DSP_inter is

signal A,B,a_temp,one_a_temp,P_0_temp,P_1_temp: std_logic_vector(7 downto 0) := (Others =>'0');

signal C,temp0,temp1:std_logic_vector(15 downto 0) := (Others =>'0');

signal clk_2,add_m: std_logic := '0'; --clk x 2, internal adder mode

component clk_x_2x is

port (

clk_2x : out STD_LOGIC;

clk_x : in STD_LOGIC

);

end component;

attribute use_dsp : string;

attribute use_dsp of A,B,C : signal is "yes";

attribute use_dsp of temp0 : signal is "yes";

attribute use_dsp of temp1 : signal is "yes";

begin

clk_io: clk_x_2x

Port Map( clk_2x => clk_2,

clk_x => clk);

reg_io:process(clk)--register inputs

begin

if(clk'event AND clk='1') then

a_temp <= alpha;

one_a_temp<=one_alpha;

P_0_temp<=P_0;

P_1_temp<=P_1;

P_f<=temp1;

end if;

end process;

add_mode: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

add_m<= NOT add_m;

end if;

end process;

dsp_proc: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

temp0<=A*B;

temp1<=temp0+C;

end if;

end process;

----muxes

A <=       P_0 when add_m='0' else

P_1 when add_m='1' ;

B <=       alpha when add_m='0' else

one_alpha when add_m='1' ;

C <=        (Others =>'0') when add_m='0' else

temp1 when add_m='1' ;

end Behavioral;

- Lab3_PartIII.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_signed.all;

entity DSP_inter is

Port(P_0: in STD_LOGIC_VECTOR (7 downto 0); --first pixel data stream input

P_1: in STD_LOGIC_VECTOR (7 downto 0); --second pixel data stream input

alpha: in STD_LOGIC_VECTOR (7 downto 0); --blending factor

one_alpha: in STD_LOGIC_VECTOR(7 downto 0);-- inverse blending factor

clk: in STD_LOGIC;

P_f: out STD_LOGIC_VECTOR(15 downto 0));--Output equal to :: P_0*alpha + P_1*one_alpha

end DSP_inter;

architecture Behavioral of DSP_inter is

signal A,B,a_temp,one_a_temp,P_0_temp,P_1_temp: std_logic_vector(7 downto 0) := (Others =>'0');

signal C,temp0,temp1:std_logic_vector(15 downto 0) := (Others =>'0');

signal clk_2,add_m: std_logic := '0'; --clk x 2, internal adder mode

component clk_x_2x is

port (

clk_2x : out STD_LOGIC;

clk_x : in STD_LOGIC

);

end component;

attribute use_dsp : string;

attribute use_dsp of A,B,C : signal is "yes";

attribute use_dsp of temp0 : signal is "yes";

attribute use_dsp of temp1 : signal is "yes";

begin

clk_io: clk_x_2x

Port Map( clk_2x => clk_2,

clk_x => clk);

reg_io:process(clk)--register inputs

begin

if(clk'event AND clk='1') then

a_temp <= alpha;

one_a_temp<=one_alpha;

P_0_temp<=P_0;

P_1_temp<=P_1;

P_f<=temp1;

end if;

end process;

add_mode: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

add_m<= NOT add_m;

end if;

end process;

dsp_proc: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

temp0<=A*B;

temp1<=temp0+C;

end if;

end process;

----muxes

A <=       P_0 when add_m='0' else

P_1 when add_m='1' ;

B <=       alpha when add_m='0' else

one_alpha when add_m='1' ;

C <=        (Others =>'0') when add_m='0' else

temp1 when add_m='1' ;

end Behavioral;

A.4  Lab3_partIII.vhd

----------------------------------------------------------------------------------

-- Company:

-- Engineer:

--

-- Create Date: 10/08/2020 02:45:13 PM

-- Design Name:

-- Module Name: lab3_partI - Behavioral

-- Project Name:

-- Target Devices:

-- Tool Versions:

-- Description:

--

-- Dependencies:

--

-- Revision:

-- Revision 0.01 - File Created

-- Additional Comments:

--

----------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_signed.all;

use ieee.numeric_std.all;

entity DSP_inter is

Port(P_0: in STD_LOGIC_VECTOR (7 downto 0); --first pixel data stream input

P_1: in STD_LOGIC_VECTOR (7 downto 0); --second pixel data stream input

alpha: in STD_LOGIC_VECTOR (7 downto 0); --blending factor

one_alpha: in STD_LOGIC_VECTOR(7 downto 0);-- inverse blending factor

clk: in STD_LOGIC;

rst: in STD_LOGIC;

P_f: out STD_LOGIC_VECTOR(15 downto 0));--Output equal to :: P_0*alpha + P_1*one_alpha

end DSP_inter;

architecture Behavioral of DSP_inter is

signal A,B,a_temp,one_a_temp,P_0_temp,P_1_temp: std_logic_vector(7 downto 0) := (Others =>'0');

signal C,temp0,temp1:std_logic_vector(15 downto 0) := (Others =>'0');

signal clk_2,add_m: std_logic := '0'; --clk x 2, internal adder mode

--64-bit shift reg components

signal a_shift_in,a_shift_out,p_shift_in,p_shift_out: STD_LOGIC :='0';

signal clk_en: STD_LOGIC :='1';

signal count: std_logic_vector(2 downto 0) := (others => '0');

CONSTANT depth: integer := 64;

component clk_x_2x is --clocking wizard

port (

clk_2x : out STD_LOGIC;

clk_x : in STD_LOGIC

);

end component;

component shift_registers_1 is --Xilnix Synthesis Guide: Inferred 64 Bit SR

generic(

DEPTH : integer := 32

);

port(

clk : in std_logic;

clken : in std_logic;

SI : in std_logic;

SO : out std_logic

);

end component;

attribute use_dsp : string;

attribute use_dsp of A,B,C : signal is "yes";

attribute use_dsp of temp0 : signal is "yes";

attribute use_dsp of temp1 : signal is "yes";

begin

clk_io: clk_x_2x

Port Map( clk_2x => clk_2,

clk_x => clk);

SR_64_P:shift_registers_1

Generic Map(

DEPTH => depth)

Port Map( clk => clk,

clken=> clk_en,

SI =>p_shift_in,

SO=>p_shift_out);

SR_64_alpha:shift_registers_1

Generic Map(

DEPTH => depth)

Port Map( clk => clk,

clken=> clk_en,

SI =>a_shift_in,

SO=>a_shift_out);

alpha_shift_io: process(clk,rst)

begin

if(rst='0') then

if(clk'event AND clk='1') then

case count is

when "111" =>

a_shift_in <= alpha(0);

p_shift_in <= P_0(0);

a_temp(7) <= a_shift_out;

P_0_temp(7)<= p_shift_out;

when "000" =>

a_shift_in <= alpha(1);

p_shift_in <= P_0(1);

a_temp(0) <= a_shift_out;

P_0_temp(0)<= p_shift_out;

when "001" =>

a_shift_in <= alpha(2);

p_shift_in <= P_0(2);

a_temp(1) <= a_shift_out;

P_0_temp(1)<= p_shift_out;

when "010" =>

a_shift_in <= alpha(3);

p_shift_in <= P_0(3);

a_temp(2) <= a_shift_out;

P_0_temp(2)<= p_shift_out;

when "011" =>

a_shift_in <= alpha(4);

p_shift_in <= P_0(4);

a_temp(3) <= a_shift_out;

P_0_temp(3)<= p_shift_out;

when "100" =>

a_shift_in <= alpha(5);

p_shift_in <= P_0(5);

a_temp(4) <= a_shift_out;

P_0_temp(4)<= p_shift_out;

when "101" =>

a_shift_in <= alpha(6);

p_shift_in <= P_0(6);

a_temp(5) <= a_shift_out;

P_0_temp(5)<= p_shift_out;

when "110" =>

a_shift_in <= alpha(7);

p_shift_in <= P_0(7);

a_temp(6) <= a_shift_out;

P_0_temp(6)<= p_shift_out;

when others =>

a_shift_in<='0';

p_shift_in<='0';

end case;

count <= count +"001";

end if;

end if;

end process;

reg_io:process(clk)--register inputs

begin

if(clk'event AND clk='1') then

one_a_temp<=one_alpha;

P_1_temp<=P_1;

P_f<=temp1;

end if;

end process;

add_mode: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

add_m<= NOT add_m;

end if;

end process;

dsp_proc: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

temp0<=A*B;

temp1<=temp0+C;

end if;

end process;

----muxes

A <=       P_0_temp when add_m='0' else

P_1_temp when add_m='1' ;

B <=       a_temp when add_m='0' else

one_a_temp when add_m='1' ;

C <=        (Others =>'0') when add_m='0' else

temp1 when add_m='1' ;

end Behavioral;

A.5  tb_Lab3_partIII.vhd

----------------------------------------------------------------------------------

-- Company:

-- Engineer:

--

-- Create Date: 10/08/2020 03:14:49 PM

-- Design Name:

-- Module Name: tb_lab3_PartI - Behavioral

-- Project Name:

-- Target Devices:

-- Tool Versions:

-- Description:

--

-- Dependencies:

--

-- Revision:

-- Revision 0.01 - File Created

-- Additional Comments:

--

----------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use ieee.math_real.all;

use std.textio.all;

use ieee.std_logic_textio.all;

use ieee.std_logic_unsigned.all;

entity tb_DSP_inter is

--  Port ( );

end tb_DSP_inter;

architecture Behavioral of tb_DSP_inter is

--Constant Definitions

CONSTANT CP: TIME := 20ns;--50 MHz clock period

CONSTANT ALPHA_VAL: real := 0.00;--blend value

CONSTANT ONE_ALPHA_VAL: real := 1.00-ALPHA_VAL; --blend value inverse

CONSTANT A_LEN: integer := 8;       --blend value bit length

CONSTANT P_LEN: integer := 8;       --pixel input value bit length

CONSTANT SHIFT_VAL: real := 2**real(A_LEN-1)-0.01; --real value representing the # of bits to shift left (arithmetic shift with real inputs)

--not synthesizable, excludes sign bit shift

--Signal Definitions

signal p0,p1: std_logic_vector(7 downto 0) := (Others =>'0');

signal a,a_chg: std_logic_vector(7 downto 0) := (Others =>'0'); --amount ot change alpha by

signal a1,a1_chg: std_logic_vector(7 downto 0) := (Others =>'0'); --amount to change one allpha by

signal pf: std_logic_vector(15 downto 0) := (Others =>'0');

SIGNAL PULSE: TIME:=CP*0.5;--pulse width

signal m_clk: std_logic := '0'; --master clock

signal reset: std_logic := '1'; -- reset

signal clk_cnt,chg_cnt: integer := 0; --used to read in files, gradually change alpha in Part I test bench

--Counts clk changes; used in Part III test bench

signal w: STD_LOGIC:= '0';

Component DSP_inter

Port(P_0: in STD_LOGIC_VECTOR (7 downto 0); --first pixel data stream input

P_1: in STD_LOGIC_VECTOR (7 downto 0); --second pixel data stream input

alpha: in STD_LOGIC_VECTOR (7 downto 0); --blending factor

one_alpha: in STD_LOGIC_VECTOR(7 downto 0);-- inverse blending factor

clk: in STD_LOGIC;

rst: in STD_LOGIC;

P_f: out STD_LOGIC_VECTOR(15 downto 0));--Output equal to :: P_0*alpha + P_1*one_alpha

end component;

begin

uut: DSP_inter--Signal Stream Blender

Port Map( P_0 => p0,

P_1 => p1,

alpha =>a,

one_alpha=>a1,

clk => m_clk,

rst => reset,

P_f =>pf);

--Clock Processs

m_clock:process --free running clock

begin

m_clk <= '1';

wait for PULSE ;

m_clk <= '0';

wait for PULSE;

end process;

r:process --reset for clock sync (~760ns clock delay @ 50 MHZ/ 100 MHz sync)

begin

wait for CP*64;

reset<= '0';

wait;

end process;

clk_count: process(m_clk,reset)

begin

if(reset='0') then

if(m_clk'event AND m_clk='1') then

clk_cnt<= clk_cnt+1;

chg_cnt<=chg_cnt+1;

if(chg_cnt=7) then

chg_cnt<=0;

end if;

end if;

end if;

end process;

--Test Vectors

process

begin

a1<="01000000";

a<="01000000";

wait;

end process;

P0_read_in:process(reset,chg_cnt,m_clk)--reads in data for P0

file TVs : text open read_mode is "P0_in.txt";-- "8_mhz_dat.txt"; --16_mhz_dat.txt -- 24_mhz_dat.txt

variable row_cnt: integer := 0;--row to read from

variable read_in     : line;

variable read_from_line : integer;

variable r_temp : std_logic_vector(7 downto 0);

begin

if(reset='0') then

if(chg_cnt=7) then

if(m_clk'event AND m_clk='1') then

if(not endfile(TVs)) then

row_cnt := row_cnt + 1;

readline(TVs,read_in); --read line from file

end if;

read(read_in,read_from_line); --read read line data from file into bit array

read(read_in,r_temp); --read read line data from file into bit array a

P0   <= std_logic_vector(to_signed(read_from_line,P_LEN));

end if;

end if;

end if;

end process;

P1_read_in:process(reset,chg_cnt,m_clk)

file TVs : text open read_mode is "P1_in.txt";-- "8_mhz_dat.txt"; --16_mhz_dat.txt -- 24_mhz_dat.txt

variable row_cnt: integer := 0;--row to read from

variable read_in: line;

variable read_from_line : integer;

variable r_temp : std_logic_vector(7 downto 0);

begin

if(reset='0') then

if(chg_cnt=7) then

if(m_clk'event AND m_clk='1') then

if(not endfile(TVs)) then

row_cnt := row_cnt + 1;

readline(TVs,read_in); --read line from file

end if;

read(read_in,read_from_line); --read read line data from file into bit array

read(read_in,r_temp); --read read line data from file into bit array a

P1  <= std_logic_vector(to_signed(read_from_line,P_LEN));

end if;

end if;

end if;

end process;

Pf_write:process (reset,m_clk)

CONSTANT COL: INTEGER :=1;

type t_integer_array is array(integer range <> )  of integer;

file Output:text open write_mode is "Pf_out.txt";

variable write_in: line; --reads in a line from the text file

variable write_line_to_file: t_integer_array(1 to COL);

begin

if(reset='0') then

if(m_clk'event AND m_clk='1') then

write_line_to_file(1):= to_integer(signed(pf));

write(write_in,write_line_to_file(1));

writeline(Output, write_in);

end if;

end if;

end process;

end Behavioral;

A.6  Shift_Registers_1.vhd

-- 32-bit Shift Register

-- Rising edge clock

-- Active high clock enable

-- foor loop-based template

-- File: shift_registers_1.vhd

library ieee;

use ieee.std_logic_1164.all;

entity shift_registers_1 is

generic(

DEPTH : integer := 32

);

port(

clk : in std_logic;

clken : in std_logic;

SI : in std_logic;

SO : out std_logic

);

end shift_registers_1;

architecture archi of shift_registers_1 is

signal shreg : std_logic_vector(DEPTH - 1 downto 0);

begin

process(clk)

begin

if rising_edge(clk) then

if clken = '1' then

for i in 0 to DEPTH - 2 loop

shreg(i + 1) <= shreg(i);

end loop;

shreg(0) <= SI;

end if;

end if;

end process;

SO <= shreg(DEPTH - 1);

end archi;

A.7 BRAM IMPLEMENTAION.VHD

----------------------------------------------------------------------------------

-- Company:

-- Engineer:

--

-- Create Date: 10/08/2020 02:45:13 PM

-- Design Name:

-- Module Name: lab3_partI - Behavioral

-- Project Name:

-- Target Devices:

-- Tool Versions:

-- Description:

--

-- Dependencies:

--

-- Revision:

-- Revision 0.01 - File Created

-- Additional Comments:

--

----------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_signed.all;

use ieee.numeric_std.all;

entity DSP_inter is

Port(P_0: in STD_LOGIC_VECTOR (7 downto 0); --first pixel data stream input

P_1: in STD_LOGIC_VECTOR (7 downto 0); --second pixel data stream input

alpha: in STD_LOGIC_VECTOR (7 downto 0); --blending factor

one_alpha: in STD_LOGIC_VECTOR(7 downto 0);-- inverse blending factor

clk: in STD_LOGIC;

rst: in STD_LOGIC;

P_f: out STD_LOGIC_VECTOR(15 downto 0));--Output equal to :: P_0*alpha + P_1*one_alpha

end DSP_inter;

architecture Behavioral of DSP_inter is

signal A,B,a_temp,one_a_temp,P_0_temp,P_1_temp: std_logic_vector(7 downto 0) := (Others =>'0');

signal C,temp0,temp1:std_logic_vector(15 downto 0) := (Others =>'0');

signal clk_2,add_m: std_logic := '0'; --clk x 2, internal adder mode

--64-bit shift reg components

signal a_shift_in,a_shift_out,p_shift_in,p_shift_out: STD_LOGIC :='0';

signal en: STD_LOGIC :='1';

signal count: std_logic_vector(6 downto 0) := (others => '0');

CONSTANT depth: integer := 64;

component clk_x_2x is --clocking wizard

port (

clk_2x : out STD_LOGIC;

clk_x : in STD_LOGIC

);

end component;

component blk_mem_gen_0 is

port (

doutb : out STD_LOGIC_VECTOR ( 7 downto 0 );

clkb : in STD_LOGIC;

clka : in STD_LOGIC;

enb : in STD_LOGIC;

ena : in STD_LOGIC;

addrb : in STD_LOGIC_VECTOR ( 5 downto 0 );

addra : in STD_LOGIC_VECTOR ( 5 downto 0 );

dina : in STD_LOGIC_VECTOR ( 7 downto 0 );

dinb:  in STD_LOGIC_VECTOR ( 7 downto 0 );

wea : in STD_LOGIC_VECTOR ( 0 to 0 )

);

end component;

attribute use_dsp : string;

attribute use_dsp of A,B,C : signal is "yes";

attribute use_dsp of temp0 : signal is "yes";

attribute use_dsp of temp1 : signal is "yes";

begin

clk_io: clk_x_2x

Port Map( clk_2x => clk_2,

clk_x => clk);

bram:blk_mem_gen_0

Port Map(

douta =>a_temp;

doutb =>P_0_temp;

clkb => clk;

clka => clk

enb => en;

ena=>en;

addrb => count;

addra => count;

dina : alpha

dinb:  P_0;

wea : 2);

reg_io:process(clk)--register inputs

begin

if(clk'event AND clk='1') then

one_a_temp<=one_alpha;

P_1_temp<=P_1;

P_f<=temp1;

end if;

end process;

add_mode: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

add_m<= NOT add_m;

end if;

end process;

dsp_proc: process(clk_2)

begin

if(clk_2'event AND clk_2='1') then

temp0<=A*B;

temp1<=temp0+C;

end if;

end process;

----muxes

A <=       P_0_temp when add_m='0' else

P_1_temp when add_m='1' ;

B <=       a_temp when add_m='0' else

one_a_temp when add_m='1' ;

C <=        (Others =>'0') when add_m='0' else

temp1 when add_m='1' ;

end Behavioral;