# Kyle Keislar

# ECE524/L FPGA/ASIC Design and Optimization Using VHDL Lab

# Lab 4

# Datapath & Control Path

# 10/16/2020

## Introduction & Problem Statements

In this experiment an averaging circuit was designed to compute the average of a specific number of integer elements. The averaging circuit would receive a serial data stream from the Block Memory, compute the running average of all the passed elements, proceeding from a pre-assigned memory address until all of the elements have been read. In Fig 4.0.A, the high level Datapath diagram for the design is shown. This design assumed 32, 8-bit inputs were provided from the block ram. All elements were assumed to be in sequential memory addresses within the BRAM starting at address 25.

Fig 4.0.A: Datapath capable of computing the average of 32 elements of an array stored in memory

To regulate the averaging circuit operation, a Finite State Machine (FSM) was required. The design in Fig 4.0.A inferred 4 registered signals, each with two control signals in addition to one control signal output that would signify all elements have been read. From this information, a High Level diagram of the Averaging Circuit FSM was synthesized. This diagram can be seen below in Fig 4.0.B.

Fig 4.0.B: Averaging Finite State Machine High-Level Diagram

Using each of the signals above as a control signal, additional signal logic would be required to regulate which state the averaging circuit was operating in. These states would be regulated by i_It32, a control signal which would be active high when 32 elements had been read. Each registered signal of the computed average (avg), the memory index to be read from the BRAM (i), the sum to be averaged (sum) and the next element to be summated (a) were supplied with an appropriate asynchronous clear and synchronous load.

The code for the synthesized design of each of these files can be found in the Appendix below.

An implemented design of this filter was not realized using the Zedboard for this experiment.

## Procedure:

Part I

Task 1: Implement the complete design including datapath and control path. Simulate your design for correct functionality. Include simulation of your design in your final report. Your simulation should clearly show your work and the average calculated by your datapath.

The design files for Task 1 can be seen in the Appendix as items A.2, A.3, A.4 and A.5. The test bench for this task can be seen as item A.1.

Task 2: Modify the design to indicate overflow. Prove that your logic works by including the proper testbench.

The testbench for this task can be seen in the appendix as item A.6.

Task 3: Modify your design if there are 50 numbers. How will the block diagram change? You need to add a logic to calculate the average of 50 numbers. Prove that your logic works by including the proper testbench.

The altered design files for the 50 element implementation can be seen in the appendix as items A.7 and A.8.

## Results (Data):

## Waveforms graphed from generated & simulated data:

Fig 4.1: Task 1 Initialization

Fig 4.2: Task 1 Output

Fig 4.3: Task II Initialization

Fig 4.4: Task II Overflow Detected

Fig 4.5: Task II Average Computation Demonstrating Overflow

Fig 4.6: Task III Initialization

Fig 4.7: Task III Average Computed

Fig 4.8: Task III Average Reset

Fig 4.9: Task III Erroneous Average

## Analysis:

The simulated design for this experiment successfully demonstrates the functionality of the averaging circuit. However, this functionality is compromised by severe design latency, and erroneous persistent values which require a complete reinitialization of all design components, including BRAM, in order to demonstrate correct functionality.

Figures 4.1-4.2 demonstrate the successful functionality of the averaging circuit under default conditions. For this task the average of the first 31 decimal numbers (0-31) were computed. The result was 15, which is the integer truncation of the actual average 15.5. No dividing logic was utilized during this implementation and the precision of the input was preserved. This implementation has a delay of 4 clock cycles after 32 elements have been read from BRAM. The total latency of the design is 36 clock cycles.

Figures 4.3-4.6 demonstrate the successful functionality of the averaging circuit with overflow detection. The overflow detection will require an additional output logic from the sum register to detect when an overflow is occurred. To facilitate this logic, the register width must be increased by 1 bit to a total of 14 data bits. The extra bit serves as an overflow detection bit. The functionality of this detection module and the behavior of the circuit under overflow conditions are shown.

Finally Figures 4.7-4.9 show a partial functionality of the 50 element averaging circuit. This circuit has will compute erroneous values on each set of inputs after the first set. This is due to persistent values on the top level of the design. The block diagram located in Figure 4.0.A must be modified by comparing the element index signal I to a constant of 50 rather than 32. In addition, the shift left operation must be replaced by a divide module whose inputs are 50 and the sum.

APPENDIX

Questions

- 1)  Estimate the FPGA area in terms of LUTs, FFs, memory, etc. for the design shown in Figure 4.1 and elaborate your answer.

Resource Usage Estimation for 32 Element Averager

For each registered signal, one flip flop/ bit is required. Given the 8-bit data elements, signal a will also be 8-bits wide and require 8 FF. The averaged output, signal avg, will also be 8 bits and will also require 8 additional FF. Due to the design constraints of the 32 Element Averaging circuit, division implementations were constrained to a shift-left operation. This implied that the pre-divided sum (the dividend of the operation) would possesses a minimum of 13 data bits during accumulation. Therefore 13 Flip Flops were allocated to these registers. Finally, to address all memory indices required in parts I, II and III 6 bits of data were required to address the BRAM used in this experiment, resulting in an additional 6 FF.

6 total LUT4s are proposed to realize this implementation and are allocated as LUTs A, B, C, D, E and F.

LUTs A and B are used to compute the next relative element indicie with inputs from register i, and construct the control signal i_It32 with inputs from register i. Each LUT has are 3 inputs provided for 4 bits from signals of the i register, the maximum indice value (32) and the address increment value (1). 2 LUTs are required to represent all 6 bits of these signals.

LUTs C and D are used to compute the next memory address to be read with inputs from register i. Each LUT has are 4 inputs provided for 4 bits from signals of the i register and the starting memory index value (25). 2 LUTs are required to represent all 6 bits of these signals.

LUTs E and F are used to compute the next summated value form registers sum and a, as well as the next running average value derived from the shifted output of register sum. Each LUT has are 3 inputs provided for 4 bits from signals of the sum register, the a register and the shifting value.  2 LUTs are required to represent all 8 bits of these signals.

Finally, 1 512x32 single port BRAM primitive was instantiated by the Vivado 2019.2 synthesis tool to satisfy the memory requirements of this experiment. These dimensions were chosen for the minimum area which would hold all of the elements.

A summation of the estimated resources is provided in the table below

| Estimated Resources |  |  |
| --- | --- | --- |
| FF | LUT4 | BRAM primitives |
| 35 | 6 | 1 |

- 2) Draw the state diagram for FSM. The state machine block diagram is shown in Figure 4.0.B

- 3) What is the FPGA area utilization for your design? How does this compare with your estimation in question 1? Explain your answer.

Utilization report for 32 Element Averager

The implemented design uses many more resources than my estimate. There are a multitude of reasons for this discrepancy outside of the estimated and actualized constraints during the design’s implementation. First the number of Look Up Tables estimated was far below what was actually requisitioned by the synthesis tool. This can be explained by the number of MAC operations used by the circuit during the computing of the average. As each SLICE only provides one adder one slice must be utilized per addition or subtraction done in a single clock pulse. This was not accounted for in my estimate. In the utilization table for the implemented design, many more additions must take place in a single clock cycle than I had originally accounted for. Each of the operations above were not constrained to be solely combinational in this design’s implementation, and many were registered. These included the control signals for the FSM and BRAM which were not estimated from the Datapath diagram located in Fig 4.0.A.

- Tb_Lab_4_Top.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_Lab_4_Top is

--  Port ( );

end tb_Lab_4_Top;

architecture Behavioral of tb_Lab_4_Top is

--Constant Definitions

CONSTANT CP: TIME := 10ns;--100 MHz clock period

CONSTANT DATA_WIDTH: integer := 8;

CONSTANT ADDRESS_WIDTH: integer := 6;

CONSTANT SUM_WIDTH: integer := 13;

--Signal Definitions

SIGNAL PULSE: TIME:=CP*0.5;--pulse width

signal m_clk: std_logic := '0'; --master clock

signal bram_reset: std_logic := '0'; -- active low reset

signal avg:   std_logic_vector (DATA_WIDTH-1 downto 0) := (Others =>'0');

--Design Components

COMPONENT Lab_4_Top is

Generic(

DATA_W: INTEGER := 8;

ADDR_W: INTEGER := 6;

SUM_W: INTEGER := 13);

Port (  clk: in STD_LOGIC;

bram_on: in STD_LOGIC;

average_top: out STD_LOGIC_VECTOR(DATA_W-1 downto 0));

end COMPONENT;

begin

uut: Lab_4_Top --Averaging circuit

Generic Map(DATA_W => DATA_WIDTH,

ADDR_W=> ADDRESS_WIDTH,

SUM_W=>SUM_WIDTH)

Port Map(   clk => m_clk,

bram_on=>bram_reset,

average_top =>avg);

--Clock Processs

m_clock:process --free running clock

begin

m_clk <= '1';

wait for PULSE ;

m_clk <= '0';

wait for PULSE;

end process;

bram_r:process --Asynch reset

begin

wait for CP*2;--clear bram for 2 clock pulses to initalize a, sum and avg registers

bram_reset<='1';

wait;

end process;

end Behavioral;

- Lab_4_Top.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Lab_4_Top is

Generic(

DATA_W: INTEGER := 8;

ADDR_W: INTEGER := 6;

SUM_W: INTEGER := 13);

Port (  clk: in STD_LOGIC;

bram_on: in STD_LOGIC;

average_top: out STD_LOGIC_VECTOR(DATA_W-1 downto 0));

end Lab_4_Top;

architecture Behavioral of Lab_4_Top is

signal data_bus: STD_LOGIC_VECTOR(DATA_W-1 downto 0) := (Others =>'0');

signal addr_bus: STD_LOGIC_VECTOR(ADDR_W-1 downto 0) := (Others =>'0');

signal i_It32_l, avg_clr_l, avg_ld_l, i_clr_l, i_ld_l, sum_clr_l, sum_ld_l, a_clr_l, a_ld_l: STD_LOGIC := '0';

--BRAM

COMPONENT Lab_4_BRAM is

Generic(D_WIDTH: INTEGER := 8;

ADDR_WIDTH: INTEGER := 6);

Port(

M_data: out STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --avg data input, sent from BRAM

M_addr: in STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0);--next element address; sent to BRAM

clk:IN STD_LOGIC;

BRAM_en: IN STD_LOGIC);

end COMPONENT;

--AVG

COMPONENT Lab_4_Avg is

Generic(

D_WIDTH: INTEGER := 8;      --Data Width

ADDR_WIDTH: INTEGER := 6;   --Address Width

SUM_WIDTH: INTEGER := 13);  --Sum bit width; set by user

Port(

--CLOCK SIGNALS

clk: in STD_LOGIC;

--DATA I/0

M_data: in STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --avg data input, sent from BRAM

M_addr: out STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0);--next element address; sent to BRAM

average: out STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --computed average of N elements

--CONTROL SIGNALS

i_It32: out STD_LOGIC; --input to FSM; high if all 31 memory addresses have been addressed

avg_clr: in STD_LOGIC; --input from FSM; clear average result (avg) register

avg_ld: in STD_LOGIC; --input from FSM; load average result (avg) register (used for reset)

i_clr: in STD_LOGIC;--input from FSM; clear address index (i) register

i_ld: in STD_LOGIC;--input from FSM; load address index (i) register

sum_clr: in STD_LOGIC;--input from FSM; clear avg accumulator (sum) register

sum_ld: in STD_LOGIC;--input from FSM; load avg accumulator (sum) register (used for reset)

a_clr: in STD_LOGIC;--input from FSM; clear average input register (a) register

a_ld: in STD_LOGIC);--input from FSM; load average input register (a) register with value from BRAM

end COMPONENT;

--FSM

COMPONENT Lab_4_FSM is

Port(

clk:    in STD_LOGIC;

i_It32: in STD_LOGIC; --FSM input; high if all 31 memory addresses have been addressed

avg_clr: out STD_LOGIC; --clear average result (avg) register

avg_ld:  out STD_LOGIC; --load average result (avg) register (used for reset)

i_clr:  out STD_LOGIC;--clear address index (i) register

i_ld:   out STD_LOGIC;--load address index (i) register

sum_clr: out STD_LOGIC;--clear avg accumulator (sum) register

sum_ld:  out STD_LOGIC;--load avg accumulator (sum) register (used for reset)

a_clr: out STD_LOGIC;--clear average input register (a) register

a_ld:  out STD_LOGIC);--load average input register (a) register with value from BRAM

end COMPONENT;

begin

avg_bram:Lab_4_BRAM

Generic Map(D_WIDTH => DATA_W,

ADDR_WIDTH=> ADDR_W)

Port Map(   M_data=>data_bus,

M_addr=>addr_bus, --next element address; sent to BRAM

clk=>clk,

BRAM_en=>bram_on);

avg_logic:Lab_4_Avg

Generic Map(D_WIDTH => DATA_W,

ADDR_WIDTH=> ADDR_W,

SUM_WIDTH =>SUM_W)

Port Map(   clk=>clk,

M_data=>data_bus,

M_addr=>addr_bus,

average=>average_top,

i_It32=>i_It32_l,

avg_clr=>avg_clr_l,

avg_ld=>avg_ld_l,

i_clr=>i_clr_l,

i_ld=>i_ld_l,

sum_clr=>sum_clr_l,

sum_ld=>sum_ld_l,

a_clr=>a_clr_l,

a_ld=>a_ld_l);

avg_fsm: Lab_4_FSM

Port Map(

clk=>clk,

i_It32=>i_It32_l,

avg_clr=>avg_clr_l,

avg_ld=>avg_ld_l,

i_clr=>i_clr_l,

i_ld=>i_ld_l,

sum_clr=>sum_clr_l,

sum_ld=>sum_ld_l,

a_clr=>a_clr_l,

a_ld=>a_ld_l);

end Behavioral;

- Lab_4_Avg.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity Lab_4_Avg is

Generic(

D_WIDTH: INTEGER := 8;      --Data Width

ADDR_WIDTH: INTEGER := 6;   --Address Width

SUM_WIDTH: INTEGER := 13);  --Sum bit width; set by user

Port(

--CLOCK SIGNALS

clk: in STD_LOGIC;

--DATA I/0

M_data: in STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --avg data input, sent from BRAM

M_addr: out STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0);--next element address; sent to BRAM

average: out STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --computed average of N elements

--CONTROL SIGNALS

i_It32: out STD_LOGIC; --input to FSM; high if all 31 memory addresses have been addressed

avg_clr: in STD_LOGIC; --input from FSM; clear average result (avg) register

avg_ld: in STD_LOGIC; --input from FSM; load average result (avg) register (used for reset)

i_clr: in STD_LOGIC;--input from FSM; clear address index (i) register

i_ld: in STD_LOGIC;--input from FSM; load address index (i) register

sum_clr: in STD_LOGIC;--input from FSM; clear avg accumulator (sum) register

sum_ld: in STD_LOGIC;--input from FSM; load avg accumulator (sum) register (used for reset)

a_clr: in STD_LOGIC;--input from FSM; clear average input register (a) register

a_ld: in STD_LOGIC);--input from FSM; load average input register (a) register with value from BRAM

end Lab_4_Avg;

architecture Behavioral of Lab_4_Avg is

CONSTANT START_ADDR: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := std_logic_vector(to_unsigned(25,ADDR_WIDTH)); --starting address of 25

CONSTANT NXT_i: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := std_logic_vector(to_unsigned(1,ADDR_WIDTH)); --amount to incremt each indicie by

signal i_count: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := (Others =>'0'); --counts memory indicies during averaging

signal a_out: STD_LOGIC_VECTOR(D_WIDTH-1 downto 0) := (Others =>'0');--data read from a reg

signal sum_in,sum_out: STD_LOGIC_VECTOR(SUM_WIDTH-1 downto 0) := (Others =>'0');--data read into sum reg from adder,

--data read from sum reg to adder

signal i_in,i_out: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := (Others =>'0');--data read from counter to index register

signal avg_in,avg_out: STD_LOGIC_VECTOR(D_WIDTH-1 downto 0) := (Others =>'0'); --data into address register

begin

a_ld_clr: process (clk,a_clr,a_ld)

begin

if(a_clr='1') then

a_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(a_ld='1') then

a_out<=M_data; --load next value form BRAM

end if;

end if;

end process;

sum_ld_clr: process (clk,sum_clr,sum_ld)

begin

if(sum_clr='1') then

sum_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(sum_ld='1') then

sum_out<=sum_in; --load next value form BRAM

end if;

end if;

end process;

i_ld_clr: process (clk,i_clr,i_ld)

begin

if(i_clr='1') then

i_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(i_ld='1') then

i_out<=i_in; --load next value form BRAM

end if;

end if;

end process;

avg_ld_clr: process (clk,avg_clr,avg_ld)

begin

if(avg_clr='1') then

avg_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(avg_ld='1') then

avg_out<=avg_in; --load next value form BRAM

end if;

end if;

end process;

--OUTPUT LOGIC

average<=avg_out;

M_addr<=i_out+START_ADDR;--output next element address

i_It32<= '1' when i_out>std_logic_vector(to_unsigned(31,ADDR_WIDTH-1)) else

'0';

--SIGNAL LOGIC

--SUM_IN

sum_in<=sum_out+a_out;

--AVG_IN

avg_in<=sum_out(SUM_WIDTH-1 downto 5); --shift left by 5 bits to divide by 32 elements

--I_IN

i_in<=i_out+NXT_i;

end Behavioral;

- Lab_4_FSM.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity Lab_4_FSM is

Port(

clk:    in STD_LOGIC;

i_It32: in STD_LOGIC; --FSM input; high if all 31 memory addresses have been addressed

avg_clr: out STD_LOGIC; --clear average result (avg) register

avg_ld:  out STD_LOGIC; --load average result (avg) register (used for reset)

i_clr:  out STD_LOGIC;--clear address index (i) register

i_ld:   out STD_LOGIC;--load address index (i) register

sum_clr: out STD_LOGIC;--clear avg accumulator (sum) register

sum_ld:  out STD_LOGIC;--load avg accumulator (sum) register (used for reset)

a_clr: out STD_LOGIC;--clear average input register (a) register

a_ld:  out STD_LOGIC);--load average input register (a) register with value from BRAM

end Lab_4_FSM;

architecture Behavioral of Lab_4_FSM is

CONSTANT AMT: INTEGER :=0;

type state is (start,cleared,acc_avg,compute);--state types: begin averaging

signal cur_st: state ; --current state, next state; defaults to start

signal nxt_st: state:=start ; --current state, next state; defaults to start

signal out_count: STD_LOGIC_VECTOR(4 downto 0) := std_logic_vector(to_unsigned(32-AMT,5));--hold output when done to compute average

signal flag,test: STD_LOGIC := '0';

begin

Change_st:process(clk) --get next state when clocked

begin

if(clk'event AND clk='1')then

cur_st  <= nxt_st;

if(cur_st = acc_avg OR cur_st=compute) then

out_count<=out_count+1;

else

out_count<= (Others =>'0');

end if;

end if;

end process;

St_ctrl: process(cur_st, i_It32,flag) --state controller

begin

case (cur_st) is

--CLEAR

when cleared =>

nxt_st  <= acc_avg;

--ACC_AVG

when acc_avg =>

if(i_It32 = '1')then

nxt_st<= compute;

else

nxt_st<= acc_avg;

end if;

when compute =>

if(flag='1') then

nxt_st<=cleared;

else

nxt_st<=compute;

end if;

when start => --return from start

nxt_st  <= cleared;

when others =>

nxt_st  <= cleared;

end case;

end process;

Out_ctrl:process(cur_st)--output controller, sets control signals based on current stae value

begin

case (cur_st) is ---Select output based on current state

--CLEAR

when cleared =>

avg_clr<='1';--clear values

i_clr<='1';

a_clr<='1';

sum_clr<='1';

--ACC_AVG

when acc_avg =>

if(i_It32 = '1')then --if all elements have been averaged

i_ld<='0';--task is done, stop loading new values

a_ld<='0';

else           --if un-averaged elements remain

avg_clr<='0';--load next values

avg_ld<='1';

i_clr<='0';

i_ld<='1';

a_clr<='0';

a_ld<='1';

sum_clr<='0';

sum_ld<='1';

end if;

when compute =>

when start =>

i_clr<='1';

a_clr<='1';

when others => --clear all regs on init/error

avg_clr<='0';

i_clr<='0';

a_clr<='0';

sum_clr<='0';

end case;

end process;

flag <= '1' when out_count="00100" else -–hold for 3 clock pulses to calculate result

'0';

end Behavioral;

- Lab_4_BRAM.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Lab_4_BRAM is

Generic(D_WIDTH: INTEGER := 8;

ADDR_WIDTH: INTEGER := 6);

Port(

M_data: out STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --avg data input, sent from BRAM

M_addr: in STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0);--next element address; sent to BRAM

clk:IN STD_LOGIC;

BRAM_en: IN STD_LOGIC);

end Lab_4_BRAM;

architecture Behavioral of Lab_4_BRAM is

COMPONENT blk_mem_gen_0 IS

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(5 DOWNTO 0);--M_addr

dina : IN STD_LOGIC_VECTOR(7 DOWNTO 0); --unused (ram initilzation done with .coe file @ C:\Users\kkeis\Documents\524_workspace\Lab_4_Avg\Lab_4_Avg.srcs\BRAM_init.coe)

douta : OUT STD_LOGIC_VECTOR(7 DOWNTO 0)--M_data

);

END COMPONENT;

signal wr_en: STD_LOGIC_VECTOR(0 DOWNTO 0):= (Others =>'0'); --tie wea to low (never write to BRAM during operation

signal BRAM_d_in: STD_LOGIC_VECTOR(D_WIDTH-1 downto 0) := (others =>'0');--tie unused din to '0'

begin

avg_bram: blk_mem_gen_0

Port Map( clka => clk,

ena=> BRAM_en,

wea=>wr_en,

addra=>M_addr,

douta=>M_data,

dina=>BRAM_d_in);

end Behavioral;

- Tb_Lab_4_Top_Overflow.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.numeric_std.all;

use IEEE.std_logic_unsigned.all;

entity tb_Lab_4_Top is

--  Port ( );

end tb_Lab_4_Top;

architecture Behavioral of tb_Lab_4_Top is

--Constant Definitions

CONSTANT CP: TIME := 10ns;--100 MHz clock period

CONSTANT DATA_WIDTH: integer := 8;

CONSTANT ADDRESS_WIDTH: integer := 6;

CONSTANT SUM_WIDTH: integer := 13;

--Signal Definitions

SIGNAL PULSE: TIME:=CP*0.5;--pulse width

signal m_clk: std_logic := '0'; --master clock

signal bram_reset,Of_det: std_logic := '0'; -- active low reset, overflow detection

signal avg:   std_logic_vector (DATA_WIDTH-1 downto 0) := (Others =>'0');

signal bad_avg: std_logic:='0';--indicates bad output; high when overflow occurs

--Design Components

COMPONENT Lab_4_Top is

Generic(

DATA_W: INTEGER := 8;

ADDR_W: INTEGER := 6;

SUM_W: INTEGER := 13);

Port (  clk: in STD_LOGIC;

bram_on: in STD_LOGIC;

Of_det: out STD_LOGIC;

average_top: out STD_LOGIC_VECTOR(DATA_W-1 downto 0));

end COMPONENT;

begin

uut: Lab_4_Top --Averaging circuit

Generic Map(DATA_W => DATA_WIDTH,

ADDR_W=> ADDRESS_WIDTH,

SUM_W=>SUM_WIDTH)

Port Map(   clk => m_clk,

bram_on=>bram_reset,

Of_det=>Of_det,

average_top =>avg);

--Clock Processs

m_clock:process --free running clock

begin

m_clk <= '1';

wait for PULSE ;

m_clk <= '0';

wait for PULSE;

end process;

bram_r:process --Asynch reset

begin

wait for CP*2;--clear bram for 2 clock pulses to initalize a, sum and avg registers

bram_reset<='1';

wait;

end process;

--Overflow Processes

Overflow:process(Of_det,bram_reset)

begin

if(bram_reset='1') then

if(Of_det='1') then

bad_avg<='1';

end if;

if(Of_det='0' AND avg=0) then

bad_avg<='0';

end if;

end if;

end process;

end Behavioral;

A.7 Lab_4_FSM_50.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity Lab_4_FSM is

Port(

clk:    in STD_LOGIC;

i_It50: in STD_LOGIC; --FSM input; high if all 31 memory addresses have been addressed

avg_clr: out STD_LOGIC; --clear average result (avg) register

avg_ld:  out STD_LOGIC; --load average result (avg) register (used for reset)

i_clr:  out STD_LOGIC;--clear address index (i) register

i_ld:   out STD_LOGIC;--load address index (i) register

sum_clr: out STD_LOGIC;--clear avg accumulator (sum) register

sum_ld:  out STD_LOGIC;--load avg accumulator (sum) register (used for reset)

a_clr: out STD_LOGIC;--clear average input register (a) register

a_ld:  out STD_LOGIC);--load average input register (a) register with value from BRAM

end Lab_4_FSM;

architecture Behavioral of Lab_4_FSM is

type state is (start,cleared,acc_avg,compute);--state types: begin averaging

signal cur_st: state ; --current state, next state; defaults to start

signal nxt_st: state:=start ; --current state, next state; defaults to start

signal out_count: STD_LOGIC_VECTOR(5 downto 0) := std_logic_vector(to_unsigned(50,6));--hold output when done to compute average

signal flag, sum_clr_flag,test: STD_LOGIC := '0';

begin

Change_st:process(clk) --get next state when clocked

begin

if(clk'event AND clk='1')then

cur_st  <= nxt_st;

if(cur_st = acc_avg OR cur_st=compute) then

out_count<=out_count+1;

else

out_count<= (Others =>'0');

end if;

end if;

end process;

St_ctrl: process(cur_st, i_It50,flag) --state controller

begin

case (cur_st) is

--CLEAR

when cleared =>

nxt_st  <= acc_avg;

--ACC_AVG

when acc_avg =>

if(i_It50 = '1')then

nxt_st<= compute;

else

nxt_st<= acc_avg;

end if;

when compute =>

if(flag='1') then

nxt_st<=cleared;

else

nxt_st<=compute;

end if;

when start => --return from start

nxt_st  <= cleared;

when others =>

nxt_st  <= cleared;

end case;

end process;

Out_ctrl:process(cur_st)--output controller, sets control signals based on current stae value

begin

case (cur_st) is ---Select output based on current state

--CLEAR

when cleared =>

avg_clr<='1';--clear values

i_clr<='1';

a_clr<='1';

sum_clr<='1';

--ACC_AVG

when acc_avg =>

if(i_It50 = '1' AND sum_clr_flag='0')then --if all elements have been averaged

i_ld<='0';--task is done, stop loading new values

a_ld<='0';

else           --if un-averaged elements remain

avg_clr<='0';--load next values

avg_ld<='1';

i_clr<='0';

i_ld<='1';

a_clr<='0';

a_ld<='1';

sum_clr<='0';

sum_ld<='1';

end if;

when compute =>

when start =>

i_clr<='1';

a_clr<='1';

when others => --clear all regs on init/error

avg_clr<='0';

i_clr<='0';

a_clr<='0';

sum_clr<='0';

end case;

end process;

flag <= '1' when out_count="000111" else

'0';

sum_clr_flag <= '1' when out_count="000111" else

'0';

end Behavioral;

A.8 Lab_4_FSM_50.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.std_logic_unsigned.all;

use ieee.numeric_std.all;

entity Lab_4_Avg is

Generic(

D_WIDTH: INTEGER := 8;      --Data Width

ADDR_WIDTH: INTEGER := 6;   --Address Width

SUM_WIDTH: INTEGER := 13);  --Sum bit width; set by user

Port(

--CLOCK SIGNALS

clk: in STD_LOGIC;

--DATA I/0

M_data: in STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --avg data input, sent from BRAM

M_addr: out STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0);--next element address; sent to BRAM

average: out STD_LOGIC_VECTOR(D_WIDTH-1 downto 0); --computed average of N elements

sum_of: out STD_LOGIC; --detects overflow when computing average

--CONTROL SIGNALS

i_It50: out STD_LOGIC; --input to FSM; high if all 31 memory addresses have been addressed

avg_clr: in STD_LOGIC; --input from FSM; clear average result (avg) register

avg_ld: in STD_LOGIC; --input from FSM; load average result (avg) register (used for reset)

i_clr: in STD_LOGIC;--input from FSM; clear address index (i) register

i_ld: in STD_LOGIC;--input from FSM; load address index (i) register

sum_clr: in STD_LOGIC;--input from FSM; clear avg accumulator (sum) register

sum_ld: in STD_LOGIC;--input from FSM; load avg accumulator (sum) register (used for reset)

a_clr: in STD_LOGIC;--input from FSM; clear average input register (a) register

a_ld: in STD_LOGIC);--input from FSM; load average input register (a) register with value from BRAM

end Lab_4_Avg;

architecture Behavioral of Lab_4_Avg is

CONSTANT START_ADDR: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := std_logic_vector(to_unsigned(14,ADDR_WIDTH)); --starting address of 25

CONSTANT NXT_i: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := std_logic_vector(to_unsigned(1,ADDR_WIDTH)); --amount to incremt each indicie by

CONSTANT FIFTY: INTEGER:=50;

signal fifty_bit: STD_LOGIC_VECTOR(D_WIDTH-1 DOWNTO 0) := std_logic_vector(to_unsigned(FIFTY,D_WIDTH));--constant signal used for division

signal sum_16_bit: STD_LOGIC_VECTOR(15 DOWNTO 0) := (Others =>'0');--16 bit signal used to implement divide by 50 average

signal computed_avg: STD_LOGIC_VECTOR(23 DOWNTO 0) := (OTHERS =>'0');

signal i_count: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := (Others =>'0'); --counts memory indicies during averaging

signal a_out: STD_LOGIC_VECTOR(D_WIDTH-1 downto 0) := (Others =>'0');--data read from a reg

signal sum_in,sum_out: STD_LOGIC_VECTOR(SUM_WIDTH downto 0) := (Others =>'0');--data read into sum reg from adder,

--data read from sum reg to adder

signal i_in,i_out: STD_LOGIC_VECTOR(ADDR_WIDTH-1 downto 0) := (Others =>'0');--data read from counter to index register

signal avg_in,avg_out: STD_LOGIC_VECTOR(D_WIDTH-1 downto 0) := (Others =>'0'); --data into address register

signal t_val_in,t_val_out: STD_LOGIC := '0'; --used by div_gen_0

signal clear_div: STD_LOGIC:='0';--clears div_gen inputs on init; internal active low asynchronous reset

COMPONENT div_gen_0 IS --Divider Generator 5.1 IP BLOCK by XILNIX

--https://www.xilinx.com/support/documentation/ip_documentation/div_gen/v5_1/pg151-div-gen.pdf

PORT (

aclk : IN STD_LOGIC;

aresetn : IN STD_LOGIC;

s_axis_divisor_tvalid : IN STD_LOGIC;

s_axis_divisor_tdata : IN STD_LOGIC_VECTOR(7 DOWNTO 0);

s_axis_dividend_tvalid : IN STD_LOGIC;

s_axis_dividend_tdata : IN STD_LOGIC_VECTOR(15 DOWNTO 0);

m_axis_dout_tvalid : OUT STD_LOGIC;

m_axis_dout_tdata : OUT STD_LOGIC_VECTOR(23 DOWNTO 0)

);

END COMPONENT;

begin

div_by_50:div_gen_0

PORT MAP (

aclk =>clk,

aresetn =>clear_div,

s_axis_divisor_tvalid=>t_val_in,

s_axis_divisor_tdata => fifty_bit,

s_axis_dividend_tvalid=>t_val_in,

s_axis_dividend_tdata =>sum_16_bit,

m_axis_dout_tvalid=>t_val_out,

m_axis_dout_tdata => computed_avg);

a_ld_clr: process (clk,a_clr,a_ld,clear_div)

begin

if(a_clr='1' OR clear_div='0') then

a_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(a_ld='1') then

a_out<=M_data; --load next value form BRAM

end if;

end if;

end process;

sum_ld_clr: process (clk,sum_clr,sum_ld)

begin

if(sum_clr='1') then

sum_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(sum_ld='1') then

sum_out<=sum_in; --load next value form BRAM

end if;

end if;

end process;

i_ld_clr: process (clk,i_clr,i_ld)

begin

if(i_clr='1') then

i_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(i_ld='1') then

i_out<=i_in; --load next value form BRAM

end if;

end if;

end process;

avg_ld_clr: process (clk,avg_clr,avg_ld)

begin

if(avg_clr='1') then

avg_out<= (Others => '0');

end if;

if(clk'event AND clk='1') then

if(avg_ld='1') then

avg_out<=avg_in; --load next value form BRAM

end if;

end if;

end process;

--OUTPUT LOGIC

average<=avg_out;

M_addr<=i_out+START_ADDR;--output next element address

i_It50<= '1' when i_out>std_logic_vector(to_unsigned(50,ADDR_WIDTH)) else

'0';

--SIGNAL LOGIC

--SUM_IN

sum_in<=sum_out+a_out;

sum_of<=sum_out(SUM_WIDTH);--MSB == Overflow detection

--DIVIDEND

sum_16_bit(SUM_WIDTH-1 downto 0)<=sum_out(SUM_WIDTH-1 downto 0);

--AVG_IN

avg_in<= computed_avg(15 downto 8); --integer component of average from

--I_IN

i_in<=i_out+NXT_i;

--clear divisor register on computed average

clear_div<='0' when i_out = START_ADDR else

'1';

end Behavioral;