# Kyle Keislar

# ECE524/L FPGA/ASIC Design and Optimization Using VHDL Lab

# Lab 6

# Arithmetic

# 11/10/2020

## Introduction & Problem Statements

In this experiment a shift-right unsigned radix 4 multiplier is designed. The multiplier handles inputs of predefined lengths whose operands are a multiplicand of 10 bits and a multiplier of 6 bits. This kind of multiplier is also know as a “High Raddix” multiplier as it computes the partial products of the multiplicand by a bit pair of the multiplier. A general high level block diagram for a fundamental radix 2 multiplier is shown below in Figure 6.0.A.

Fig 6.0.A: General Schema of the High Raddix multiplier

The detailed block diagram for the multiplier designed in this experiment is shown below as Fig 6.0.B in the Procedure section below. The proposed design diagram references its general counterpart above and is comprised of two modules and one top level. A testbench was also written to demonstrate the functionality of the unsigned multiplier.

## Procedure

Task 1: Provide the detailed block diagram of your design.

Fig 6.0.B: General Schema of the High Raddix multiplier

Task 2: Provide VHDL source code and VHDL testbench for the design.

The simulated design files for Task 2 can be seen in the Appendix below as items: A.2 Lab_6_Umult.vhd, A.3 dw_pp.vhd and A.4 SR_rad4.vhd. The test bench utilized for Task 2, can be found as A.1 tb_Lab_6_Umult.vhd.

## Results (Data):

## Waveforms graphed from generated & simulated data:

Fig 6.1: Radix 4 Multiplier Test Vector products 1,2

Fig 6.2: Radix 4 Multiplier Test Vector products 3,4

## Analysis:

The simulated design for this experiment successfully demonstrated the functionality of the radix 4 unsigned multiplier proposed in Fig 6.0.B above.

Figure 6.1 and 6.2 show 4 collective test vector cases with different 10 bit multiplicands and 6 bit multipliers. In the first figure, the test vectors for the first two pairs and their products are shown. The first multiplier input, x, and multiplicand input a are 638 and 33 respectively. The expected result is 638 * 33 = 22,539. This value can be seen in the simulation waveform above after the design latency of 7 clock pulses following the reset signal. The next case illustrates the multiplier functionality with a smaller multiplier whose binary signal contains the bit pair “11”. Here the multiplicand of 9 is multiplied by 15 and the result of 135 can be seen 7 clock cycles after the inputs are reset at 150 ns.

In the second figure, the test vectors for the last two pairs and their products are shown. The first multiplier and multiplicand pair is 512 and 1. The expected result is 512  and can be seen after the design latency at 230 ns. This test vector was chosen to illustrate a fundamental property of multiplication when one of the operands is 1 and the other is non-zero. The next case illustrates the multiplier behavior when one of the operands is 0. Here, the multiplier is 0 but the multiplicand is increased to its maximum value of 1023. The result is 0 as expected and can be seen present in the waveform at 310 ns.

These results confirm the basic functionality of the multiplier for the minimum and maximum bounds of testing. This design could be improved greatly by utilizing additional FPGA resources such as a DSP block. This method was not pursued in this experiment as no realized design was implemented. This would also reduce the latency of the design as multiple additions could be performed in a single clock cycle without utilizing any additional resources. More extensive testing will be required to utilize this multiplier in future designs.

APPENDIX

- tb_Lab_6_Umult.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use ieee.numeric_std.all;

use ieee.std_logic_unsigned.all;

entity tb_Lab_6_Umult is

--  Port ( );

end tb_Lab_6_Umult ;

architecture Behavioral of tb_Lab_6_Umult is

--CONSTANT DEFINITIONS

CONSTANT X_W: INTEGER := 6; --multiplier width

CONSTANT A_W: INTEGER := 10; --multiplicand width

CONSTANT RADDIX: INTEGER :=4; --raddix used to compute TB_RAD_BITS

CONSTANT TB_RAD_BITS: INTEGER := 2;

--signal defintions

signal A: STD_LOGIC_VECTOR(A_W-1 DOWNTO 0) := (Others =>'0');

signal X: STD_LOGIC_VECTOR(X_W-1 downto 0) := (Others =>'0');

signal Product: STD_LOGIC_VECTOR((2*A_W)-1 downto 0) := (Others =>'0');

--CLOCK process signal definitiions

CONSTANT CP: TIME := 10ns;--clock period

SIGNAL PULSE: TIME:=CP*0.5;--pulse width

signal m_clk,reset: std_logic := '0'; --master clock, hold initial values

--Component Definitions

COMPONENT Lab_6_Umult is

GENERIC(A_WIDTH: INTEGER:=4;-- shift register bit width

X_WIDTH: INTEGER:=4;

RAD_BITS: INTEGER :=2); --Raddix of multiplier

Port ( a: in STD_LOGIC_VECTOR (A_WIDTH-1 downto 0); --multipler, multiplicand

X: in STD_LOGIC_VECTOR(X_WIDTH-1 downto 0);

clk,rst : in STD_LOGIC;

P : out STD_LOGIC_VECTOR ((A_WIDTH*2)-1 downto 0)); --proudct

end COMPONENT;

begin

uut:Lab_6_Umult

Generic Map ( A_WIDTH => A_W,

X_WIDTH => X_W,

RAD_BITS => TB_RAD_BITS)

Port Map    ( a=>A,

x=>X,

clk=>m_clk,

rst=>reset,

P=> Product);

--Clock Processs

m_clock:process --free running clock

begin

m_clk <= '1';

wait for PULSE ;

m_clk <= '0';

wait for PULSE;

end process;

--Clock Processs

rst:process --fholds initilized values to show 0 behavior

begin

reset<= '1';

wait for CP ;

reset <= '0';

wait for CP*7;

reset<= '1';

wait for CP ;

reset <= '0';

wait for CP*7;

reset<= '1';

wait for CP ;

reset <= '0';

wait for CP*7;

reset<= '1';

wait for CP ;

reset <= '0';

wait for CP*7;

wait;

end process;

TVs:process

begin

wait for CP;

A<= "1010101011"; --683 in binary

X<= "100001" ;   -- 33 in Binary

wait for CP*7;

--Expected Result:

--683 * 33 = 22,539

A<= "0000001001"; --9 in binary

X<= "001111" ;   -- 15 in Binary

wait for CP*7;

wait for CP;

--Expected Result:

--9 * 15 = 135

A<= "1000000000"; --512 in binary

X<= "000001" ;   -- 15 in Binary

wait for CP*7;

wait for CP;

--Expected Result:

--512 * 1 = 512

A<= "1111111111"; --1023 in binary

X<= "000000" ;   -- 0 in Binary

wait for CP*7;

wait for CP;

--Expected Result:

--1023 * 0 = 0

wait;

end process;

end Behavioral;

- Lab_6_Umult.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL; --Included for convient signal incrementation

entity Lab_6_Umult is

GENERIC(A_WIDTH: INTEGER:=4;-- shift register bit width

X_WIDTH: INTEGER:=4;

RAD_BITS: INTEGER :=2); --Raddix of multiplier

Port ( a: in STD_LOGIC_VECTOR (A_WIDTH-1 downto 0); --multipler, multiplicand

X: in STD_LOGIC_VECTOR(X_WIDTH-1 downto 0);

clk,rst : in STD_LOGIC;

P : out STD_LOGIC_VECTOR ((A_WIDTH*2)-1 downto 0)); --proudct

end Lab_6_Umult;

architecture Behavioral of Lab_6_Umult is

--CONSTANTS

CONSTANT CONCAT: STD_LOGIC_VECTOR(RAD_BITS-1 downto 0) := (Others => '0');

signal zero_mux: STD_LOGIC_VECTOR(11 downto 0) := (Others =>'0');

signal a_mux: STD_LOGIC_VECTOR(11 downto 0) := (Others =>'0');

signal two_a_mux: STD_LOGIC_VECTOR(11 downto 0):= (Others =>'0');

signal three_a_mux: STD_LOGIC_VECTOR(11 downto 0):= (Others =>'0');

--SIGNALS

signal X_SR_in:STD_LOGIC_VECTOR(X_WIDTH-1 DOWNTO 0):= (Others =>'0');

signal temp_x,X_SR_out:STD_LOGIC_VECTOR(X_WIDTH-1 DOWNTO 0):= (Others =>'0');          --Shift Register Output for Multplier

signal mult_sel: STD_LOGIC_VECTOR(RAD_BITS-1 downto 0) := (Others =>'0');        --input from shift Register to Adder MUX select lines

signal partial_add: STD_LOGIC_VECTOR(A_WIDTH-1 DOWNTO 0):= (Others =>'0'); --Next partial product to be summated

signal partial_in,next_partial: STD_LOGIC_VECTOR(A_WIDTH-1 DOWNTO 0):= (Others =>'0'); --Current partial sum to be stored in doublewidth partial product register

signal Add_in: STD_LOGIC_VECTOR(11 DOWNTO 0):= (Others =>'0');-- selected input based off of shifted in RAD 4, two bit pairs form X

signal Sum_out: STD_LOGIC_VECTOR(11 downto 0) := (Others =>'0'); --output sum to drive c_out, partial_in

signal c_out,count: STD_LOGIC_VECTOR(RAD_BITS-1 downto 0) := (Others =>'0');  --carryout from adder

--COMPONENTS

--Rad4 Shift REg

COMPONENT SR_rad4 is

GENERIC(WIDTH: INTEGER:=4;-- shift register bit width

RAD_BITS: INTEGER:=2); --Amount of bits to shift for Raddix

Port ( Reset,clk: in STD_LOGIC; --0 => SR 1 => SL

SR_in: in STD_LOGIC_VECTOR (WIDTH-1 downto 0);

SR_out: out STD_LOGIC_VECTOR (WIDTH-1 downto 0);

SR_mult_out: out STD_LOGIC_VECTOR(RAD_BITS-1 downto 0));

end COMPONENT;

--Double Width Partial Product Reg

COMPONENT dw_pp is

GENERIC(A_WIDTH: INTEGER:=4;

X_WIDTH: INTEGER:=4;

RAD_BITS: INTEGER:=2); -- shift register bit width

Port ( clk: in STD_LOGIC;

Reset : in STD_LOGIC;

PP_in: in STD_LOGIC_VECTOR (A_WIDTH-1 DOWNTO 0);

carry_in,count: in STD_LOGIC_VECTOR(RAD_BITS-1 downto 0) ;

Prod : out STD_LOGIC_VECTOR ((A_WIDTH*2)-1 downto 0)

);

end COMPONENT;

begin

--Component Instantiation

DBL_P: dw_pp

Generic Map( A_WIDTH => A_WIDTH,

X_WIDTH => X_WIDTH,

RAD_BITS=>RAD_BITS)

Port Map(

clk => clk,

Reset => rst,

PP_in => partial_in,

carry_in => c_out,

count=>count,

Prod => P);

SR_4: SR_rad4

Generic Map(WIDTH=>X_WIDTH,

RAD_BITS=>RAD_BITS) -- shift register bit width

Port Map ( Reset=>rst,

clk=>clk,

SR_in=>x_SR_in,

SR_out=>X_SR_out,

SR_mult_out=>mult_sel) ;

Init:process(rst,a)

begin

--after reset, define multiplicand values based on A-input

if(rst'event AND rst='0') then

a_mux <= "00"&a;--'0'& std_logic_vector(unsigned(a));

two_a_mux<= '0'&a& '0';

three_a_mux<= std_logic_vector(("00" & UNSIGNED(a)) + ('0'& UNSIGNED(a)&'0'));

end if;

end process;

Comp_sum_shift: process(clk)

begin

if(rst='1' ) then

Sum_out<=(Others=>'0');

X_SR_in<=(Others=>'0');

count <= (Others =>'1');

elsif (rst'event AND rst='0') then

X_SR_in<=x;

else

if(clk'event AND clk='1') then

--Find next partial product to add

partial_in<=Sum_Out(9 downto 0);

--compute next partial sum to store in doublewidth

if(mult_sel="00") then

Sum_out<=zero_mux+('0'&partial_add);

elsif (mult_sel="01") then

Sum_out<=a_mux+('0'&partial_add);

elsif(mult_sel="10") then

Sum_out<=two_a_mux+('0'&partial_add);

elsif(mult_sel="11") then

Sum_out<=three_a_mux+('0'&partial_add);

end if;

count<=count+1;

--shift in next multiplier bit pair

X_SR_in<= "00" & X_SR_in(X_WIDTH-1 downto RAD_BITS);

end if;

end if;

end process;

--Adder Multiplexer

--Sum_out<=(Add_in)+('0'&partial_add);

partial_add<= "00"&Sum_out(9 downto 2);

c_out<=Sum_out(11 downto 10);

with mult_sel  select Add_in <=

zero_mux when "00",

a_mux when "01",

two_a_mux when "10",

three_a_mux when "11",

Add_in when others;

end Behavioral;

- Dw_pp.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL; --Included for convient signal incrementation

entity dw_pp is

GENERIC(A_WIDTH: INTEGER:=4;

X_WIDTH: INTEGER:=4;

RAD_BITS: INTEGER:=2); -- shift register bit width

Port ( clk: in STD_LOGIC;

Reset : in STD_LOGIC;

PP_in: in STD_LOGIC_VECTOR (A_WIDTH-1 DOWNTO 0);

carry_in,count: in STD_LOGIC_VECTOR(RAD_BITS-1 downto 0) ;

Prod : out STD_LOGIC_VECTOR ((A_WIDTH*2)-1 downto 0)

);

end dw_pp;

architecture Behavioral of dw_pp is

signal temp_p: STD_LOGIC_VECTOR ((2*A_WIDTH)-1 downto 0):= (Others => '0');

signal prev_carry: STD_LOGIC_VECTOR(RAD_BITS-1 downto 0):= (Others => '0');

signal nxt_prod: STD_LOGIC_VECTOR ((A_WIDTH*2)-1 downto 0):= (Others => '0');

begin

process(clk)

begin

if(reset='1') then

Prod<= (Others =>'0');

else

if(clk'event AND clk='1') then

Prod<=nxt_prod;

prev_carry<=carry_in;

case count is

when "01" =>

temp_p(3 downto 2) <= pp_in(1 downto 0);

when "10" =>

temp_p(5 downto 4) <= pp_in(1 downto 0);

when "11" =>

temp_p(7 downto 6) <= pp_in(1 downto 0);

when others =>

temp_p<=temp_p;

end case;

end if;

end if;

end process;

nxt_prod(15 downto 0)<= prev_carry&pp_in&temp_p(7 downto 4);

end Behavioral;

- SR_rad4.vhd

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL; --Included for convient signal incrementation

entity SR_rad4 is

GENERIC(WIDTH: INTEGER:=4;-- shift register bit width

RAD_BITS: INTEGER:=2); --Amount of bits to shift for Raddix

Port ( Reset,clk: in STD_LOGIC; --0 => SR 1 => SL

SR_in: in STD_LOGIC_VECTOR (WIDTH-1 downto 0);

SR_out: out STD_LOGIC_VECTOR (WIDTH-1 downto 0);

SR_mult_out: out STD_LOGIC_VECTOR(RAD_BITS-1 downto 0));

end SR_rad4;

architecture Behavioral of SR_rad4 is

--signal SR_nxt: STD_LOGIC_VECTOR (WIDTH-1 downto 0);

signal temp,shift: STD_LOGIC_VECTOR(WIDTH-1 downto 0):= (Others => '0');

begin

process (clk,Reset)

begin

if (Reset = '1') then

SR_out <= (others => '0');

SR_mult_out <= (others =>'0');

end if;

if(rising_edge(clk) AND Reset ='0') then

SR_mult_out <= SR_in(RAD_BITS-1 downto 0);

for i in 0 to WIDTH-(RAD_BITS+1) loop

SR_out(i) <= Sr_in(i+(RAD_BITS));

end loop;

end if;

end process;

end Behavioral;